Computer Architecture Exam Notes

Question 1 – 2018 (IA32 and x64)

- CDECL used for C and C++

- Result always returned in eax and rax

- Parameters are pushed onto stack in reverse order

- Stack aligned on 4-bit and 8-bit boundary respectively

- X64 parameters pushed in rcx, rdx, r8, r9

- X64 uses 32-bytes of shadow space for parameters

- IA32 user must adjust stack

- In IA32 non-volatile registers are ebx

- In x64 non-volatile registers are rbx, r12-15

Question 2 – 2018 (Page Tables)

|  | min process (i) | max process (ii) |
| --- | --- | --- |
| 1 level page table | 4MB | 4MB |
| 2 level page table | 4KB + 4KB (1 primary level table always present, smallest process will need at least one secondary) | 4KB + 4MB (1 primary level table + 1024 secondary level tables) |

Question 3 – 2018 (Cache)

- L = Bytes per cache line

- K = Directories across

- N = Sets down

- Direct Mapped  K = 1

- Fully Associative  N = 1

| - 64 byte Direct Mapped Cache - L = 16 bytes - K = 1 - N = 4 - Offset: Last 4 bits - Set: Next 2 bits - Tag: Remaining bits | - 64 byte Fully Associative Cache - L = 16 bytes - K = 4 - N = 1 - Offset: Last 4 bits - Set: 0 BITS (only one set) - Tag: Remaining bits |
| --- | --- |